AutoLine QGIS Plugin Installation Guide

Date:15/10/2025

by Jan Majcher (janmajcher710@gmail.com)

Plugin description (more in Readme.txt)

AutoLine is a QGIS plugin for automated Multibeam and Side Scan Sonar (SSS) line planning.

Apart from set offset line spacing, it can also dynamically generate tracklines, crosslines, and coverage layers based on survey polygon and existing DEM data.

Requirements

QGIS 3.10+

Python 3.7+

Enabled QGIS Processing Toolbox

Required QGIS providers:

native:pointsalonglines

native:rastersampling

native:difference

native:offsetline

native:buffer

native:minimumboundinggeometry

native:simplifygeometries

Installation Steps

1. Copy Plugin Folder

Copy the entire folder containing:

__init__.py

metadata.txt

line_planner_plugin.py

line_planner_dialog.py

dialog_line_planner.ui

main_line_planner.py

main_line_planner_popup.py

geometry_tools.py

depth_sampling.py

line_generator.py

coverage_analysis.py

infill_generator.py

icon.svg

to your QGIS plugins directory:

Windows:

%AppData%\QGIS\QGIS3\profiles\default\python\plugins\AutoLine

Linux/macOS:

~/.local/share/QGIS/QGIS3/profiles/default/python/plugins/AutoLine

(If the folder “AutoLine” doesn’t exist — create it manually.)

2. Restart QGIS

After copying, restart QGIS.

3. Enable the Plugin

In QGIS:

Go to Plugins → Manage and Install Plugins…

Find AutoLine

Click Install or Enable

4. Launch

A new toolbar icon

labeled “Line Planner” will appear.

Click it to open the AutoLine interface.